# Software Resource Specification of # B3VCU # STELS PART NO: 2200302054

# Change Index.

| Index | Date | Amendment contents | Version | Person |
| --- | --- | --- | --- | --- |
| 1 | 14 May2022 | First Copy for release | 1.0.1 | Chen Yong |
| 2 | 3 Aug 2022 | Modify by VCU_IDS_V0.2.xlsx requirement | 1.0.2 | Chen Yong |
| 3 | 17 Aug 2022 | Modify by VDU Software V0.5_16082022.pptx | 1.0.3 | Chen Yong |

Table of Contents

1	Scope	5

1.1	System overview	5

1.2	Document overview	5

2	Requirements	5

2.1	Required states and modes	5

2.1.1	verbose information mode	5

2.1.2	Normal working mode	6

2.2	CSCI capability requirements	6

2.2.1	Booting up	6

2.2.2	Watchdog enable	6

2.2.3	Video Error monitor	6

2.3	CSCI external interface requirements	7

2.3.1	CAM UART	7

2.3.2	VDU UART	7

2.3.3	SPARE UART	7

2.3.4	Ethernet	8

2.3.5	KeyPad	9

2.3.6	Digital GPIO Output	9

2.3.7	SDI Video Input	9

2.3.8	SDI Video Output	10

2.3.9	LED	10

2.4	CSCI internal interface requirements	10

2.4.1	I2C	10

2.4.2	frmbuf_rd	11

2.4.3	mix	11

2.4.4	uhdsdi_tx	11

2.4.5	sdi_anc_tx	11

2.4.6	iio sensor	12

2.4.7	pdma	12

2.4.8	Reserv memory	12

2.4.9	Watchdog	12

2.5	CSCI internal data requirements	12

2.5.1	Log data	12

2.5.2	Configuration file data	14

2.5.3	global parameters	15

3	Rrequirements traceability	15

4	Note	15

5	Appendixes	15

# Scope

## System overview

This document stipulates Software resources specification procedures for Video Control Unit (abbreviated as VCU B3 here after). This document will be used as a guideline to system resources and programming of VCU B3 in according to the technical specification requirements.

## Document overview

Chapter 1: Scope

Chapter 2: Referenced documents

Chapter 3: Requirements

Chapter 4: Requirements traceability

Chapter 5: Notes

chapter 6: Appendixes.

# Referenced documents

# Requirements

## Required states and modes

### verbose information mode

Run application with option ‘-v’, then it will enter into the print verbose   information mode. It will print out the verbose information of the VCU and exit.

The verbose information includes: the device info, the configuration data, the error recording data and the working information data.

- The device info:(refer 3.5.2)

Main Version:x.x.x (the whole system version)

App Version:x.x.x (the firmware version)

OSD Variant:xxxx(the VDU OSD variant)

SN:B3VCU-xxx (the VCU serial Number)

date:xx-xx-xxxx (manufacturing date)

- The Configuration data: (refer 3.5.1)

Configuration: tick=xxxxx (the total power on time. unit(s))

- the error recording data:(refer 3.5.1)

Recording:

timestamp=xxxxx info=0xhhhhhhhh desc=xxxxxxxxxxx

....

-------Over----------

- the working information data:(refer 3.4.6)

Power Current: raw=xxx.xxx scale=x.xxx current=x.xxxx

PS temp:raw=xxx.xxx scale=x.xxx offset=xxx.xxx temp=xx.xxx

### Normal working mode

Defaut system booting up, the application will run in this normal working mode. The system script file is in /etc/init.d/loaduserapp.sh.

It has a configuration file. The default configuration file is /etc/b3vcu.conf. It can be assign a special configuration file by option(-f filename). The configuration file data format refer to 3.5.2

## CSCI capability requirements

### Booting up

The maxium system booting up time is less than 30 seconds.

### Watchdog enable

Enable watchdog and the application refresh the watchdog timer. The watchdog can restart the system if the application cannot refresh the watchdog.

### Video Error monitor

Keep monitoring the video input. Disable the SDI output if any error in the video input is found.

## CSCI external interface requirements

### CAM UART

Format: RS422

Device name: /dev/ttySC0

working mode:Duplex

data: 115200n8

flow control: no

The “NUC” command output to camera.

Protocol refer to document (TSP-545-AJ-0014-C_23Oct19.pdf) provided by STK

### VDU UART

Format: RS422

Device name: /dev/ttySC1

working mode:Duplex

data: 115200n8

flow control: no

Receiving “Key” information from VDU and reponse it.

Protocol refer to document (Serial Protocol) provided by Teamone

### SPARE UART

Format: RS232

Device name: /dev/ttyPS1,  /dev/ttySC2,  /dev/ttySC3

working mode:Duplex

data: 115200n8

flow control: no

### Ethernet

10/100/1000 BASE-T

- Port 1(eth2): VCU control port static IP  192.168.1.254   port 51000

- Port 2(eth0): VCU RTP video stream port. Static IP 192.168.6.254

- Port 3(eth1): VCU RTP video stream port. Static IP 192.168.7.254

RTP Video Stream:

To Host IP: Port 2: 192.168.6.255 Port 3: 192.168.7.255

To Host port 5001: Video Input 0

To Host port 5002: Video Input 1

To Host port 5003: Video Input 2

To Host port 5004: Video Input 3

To Host port 5005: Video Input 4

To Host port 5006: Video Input 5

To Host port 5007: Video Input 6

To Host port 5008: Video Input 7

To Host port 5009: Video Input 8

To Host port 5010: Video Input 9

Format:  UDP /RTP

clock rate: 90000

encoding name:H264

payload:96

Control:

Protocol Refer to document (VCU_IDS_V0.3.xlsx)

### KeyPad

| Key No | Gpio port | Key name | Linux code |  |
| --- | --- | --- | --- | --- |
| KEY_DIN_1 | <axi_gpio_0 0> | key1 | KEY_1(2) | Trailer ? |
| KEY_DIN_2 | <axi_gpio_0 1> | key2 | KEY_2(3) | Reverse Gear |
| KEY_DIN_3 | <axi_gpio_0 2> | key3 | KEY_3(4) | Forward Gear |
| KEY_DIN_4 | <axi_gpio_0 3> | key4 | KEY_4(5) | TBD |
| KEY_DIN_5 | <axi_gpio_0 4> | key5 | KEY_5(6) | TBD |
| KEY_DIN_6 | <axi_gpio_0 5> | key6 | KEY_6(7) | TBD |
| KEY_DIN_7 | <axi_gpio_0 6> | key7 | KEY_7(8) | TBD |
| KEY_DIN_8 | <axi_gpio_0 7> | key8 | KEY_8(9) | TBD |

### Digital GPIO Output

| DO No | Linux GPIO No | Terminal | Description |
| --- | --- | --- | --- |
| DO1 | 492 | IR | On/OFF IR |
| DO2 | 493 | IR | On/OFF IR |
| DO3 | 494 | TBD |  |
| DO4 | 495 | TBD |  |
| DO5 | 496 | TBD |  |

### SDI Video Input

Device name: /dev/media6 (/dev/video0)

/dev/media7 (/dev/video1)

/dev/media8 (/dev/video2)

/dev/media9 (/dev/video3)

/dev/media0 (/dev/video4)

/dev/media1 (/dev/video5)

/dev/media2 (/dev/video6)

/dev/media3 (/dev/video7)

/dev/media4 (/dev/video8)

/dev/media5 (/dev/video9)

SDI0-SDI3 support format: 1080P60 or lower resolution.

SDI4-SDI9 support format: 1080P30

### SDI Video Output

SDI0 output: frmbuf_rd --> mix -> uhdsdi_tx (‘mix’ can mix video and osd)

SDI1 – SDI7 output: frmbuf_rd->udhsdi_tx. (without osd funciton)

There is a device “sdi_anc_tx” in all output. It can embed timestamp into the SDI signal.

all devices refer to internal interface:

frmbuf_rd: refer to 3.4.2

mix: refer to 3.4.3

uhdsdi_tx: refer to 3.4.4

sdi_anc_tx: refer to 3.4.5

### LED

| LED | Linux GPIO No | Description |
| --- | --- | --- |
| Green | 458 | Normal Status |
| Red | 459 | BIT error |

## CSCI internal interface requirements

### I2C

device name: /dev/i2c-0

slave chip: mb85rc256vfp  slave address: 0x50

description: log data storage

### frmbuf_rd

Address: sdi0(0xb02d0000) sdi1(0xb02f0000) sdi2(0xb0310000) sdi3(0xb0330000) sdi4(0xb03a0000) sdi5(0xb03c0000) sdi6(0xb03e0000) sdi7(0xb0400000)

Support Format: Y_UV10, Y_UV8

refer to document (pg278-v-frmbuf.pd) provided by Xilinx

### mix

Address: 0xb02e0000

Primary video format: YUV4:2:2 10bits

Overlay layer1 format: BGRA8

Maximum resolution: 1920x1080

refer to document (pg243-v-mix.pdf) provided by Xilinx

### uhdsdi_tx

Address: sdi0(0xa0040000) sdi1(0xa0060000) sdi2(0xa0080000) sdi3(0xa00a0000) sdi4(0xa0100000) sdi5(0xa0120000) sdi6(0xa0140000) sdi7(0xa0160000)

Mode: 3G SDI 10bits

format: NV16_10LE32

refer to document (pg289-v-smpte-uhdsdi-tx-ss.pdf) provided by Xilinx

### sdi_anc_tx

Address: sdi0(0xa01c0000) sdi1(0xa01d0000) sdi2(0xa01e0000) sdi3(0xa01f0000) sdi4(0xa0200000) sdi5(0xa0210000) sdi6(0xa0220000) sdi7(0xa0230000)

data format: 64bits unsigned interger. Microseconds of the epoch time.

### iio sensor

CPU:/sys/bus/iio/devices/iio\:device0

Power Current Sensor:/sys/bus/iio/devices/iio\:device1

### pdma

It is a user interface to transfer a DAM address of the user space to the physical address.

Device: /dev/pdma0

ioctl cmd: 0

ioctl parameters: struct{guint64 param_in; guint64 param_out}

param_in: the dma user-space address.

param_out: the dma physical address

### Reserv memory

Osd reserve memory physcial address:0x70000000

osd reserve memory size: 0xff00000

osd image format: ARGB32

### Watchdog

Device: /dev/watchdog0, /dev/watchdog1

watchdog0 is controlled by OS.

Watchdog1 can be controoled by the user.

## CSCI internal data requirements

### Log data

Log data is stored in the Log FRAM chip. It includes two configuration data structrue and the error recording data array. The error recording data array can fill the space of the log chip except the configuration data area.

| Item | Type | Value | Description |
| --- | --- | --- | --- |
| conf[2] (logdata_conf_data_t) | header (guint32) | 0x5555aaaa | Configuration data header ID |
|  | timestamp (guint32) |  | Linux epoch time(unit:s) |
|  | totaltick (guint32) |  | Totoal power on time(unit:s) |
|  | rev[2] (guint16) |  | Reserve data space |
|  | chk (guint16) |  | CRC16 checksum(modbus) |
| rec[] (logdata_rec_data_t) | header (guint32) | 0x66669999 | Recording data header ID |
|  | timestamp (guint32) |  | Linux epoch time(unit:s) |
|  | infocode (guint32) |  | 0x80000001: memory error 0x80000002:temperature error 0x80000003:power error 0x80010001:log chip error 0x80010002:sensor error 0x80010003:key error 0x80010004:sdi device error 0x80010005:sdi out device error 0x80010006:osd device error 0x80010007:serial device error |
|  | desc[34] (guint8) |  | Error info description |
|  | chk (guin16) |  | CRC16 checksum(modbus) |

### Configuration file data

Default Configuration file: /etc/b3vdu.conf

file data format based on “libconfig”

| Item | Type | Value | descritpion |
| --- | --- | --- | --- |
| device.version | String | “x.x.x” | Whole system version |
| device.SN | String | “B3VCU-xxx” | Device Serial Number |
| device.date | String | “xx-xx-xxxx” | Manufacturing Date |
| VARIANT | String | “xxxxxx” | VDU OSD variant |
| BITMASK | HEX(32 bits) | 0xhhhhhhhh | Error bit mask for light 0:disable 1: enable |
| DISABEL_WATCHDOG | Boolean | true: disable watchdog false: enable watchdog | Firmware watchdog enable/disable |
| DISABLE_SEI | Boolean | true: disable SEI package false: enable SEI package | SEI package of RTP/UDP streamer data |
| MaxRtpStream | Integer | 1 – 8 | The maximum rtp streams can be enable. |

### global parameters

| PGParam (glb_priv_data_t) | Sysbits (guint32) | Bit0: memory bit1: logdata bit2: snsor bit3: current bit4: temperature bit5: keypad bit6: video out0 bit7: video out1 bit8: video out2 bit9: video out3 bit10: video out4 bit11: video out5 bit12: video out6 bit13: video out7 bit14: vdieo in0 bit15: video in1 bit16: video in2 bit17: video in3 bit18: video in4 bit19: video in5 bit20: video in6 bit21: video in7 bit22: video in8 bit23: video in9 bit24: osd bit25: serial bit26: vdu | System BIT information: 0: pass 1: fail |
| --- | --- | --- | --- |

# Rrequirements traceability

# Note

# Appendixes