SDK and system resources of B3VCU

Revision History

| Name | Date | Reason For Changes | Version |
| --- | --- | --- | --- |
| Chen Yong | Jul 7, 2022 | Initial draft | 1.0.1 |

Table of Contents

1  Scope	4

2  System	4

3  Enviroment	4

3.1  VCU	4

3.2  Programming computer	5

4  B3VCU system resources	5

4.1  External interface	5

4.1.1  CAM UART	5

4.1.2  VDU UART	5

4.1.3  Spare UART	6

4.1.4  Ethernet	6

4.1.5  KeyPad	6

4.1.6  Digital GPIO Output	6

4.1.7  SDI Video Input	6

4.1.8  SDI video Output	7

4.1.9  LED	7

4.2  Internal Interface	7

4.2.1  I2C	7

4.2.2  frmbuf_rd	7

4.2.3  mix	7

4.2.4  uhdsdi_tx	8

4.2.5  sdi_anc_tx	8

4.2.6  IIo sensor	8

4.2.7  pdma	8

4.2.8  Reserve memory	8

4.2.9  Watchdog	8

5  Programming	9

5.1  A simple firmware	9

5.1.1  Edit	9

5.1.2  Compile	10

5.1.3  Run firmware	10

# Scope

# System

VCU

VDU

CAM

Display

RTP/UDP

player

Hub

VCU Control

computer

Programming

computer

Sdi output 1

RS422

SDI(1-10)

SDI output(2-8)

Ethernet 0-1

Ethernet 2

Usb debug serial port

RS422

Figure is the full system physical linking. The devices and linking for programming are in the dotted line box.

# Enviroment

## VCU

Defaut setting:

- static IP (192.168.9.115)

- username: root

- password: root

The user can download and upload files to VCU with sftp client software.

The user can control the VCU with the serial port debug application(e.g: teraterm).

The user also can log in and control the VCU with putty or ssh.

## Programming computer

- This computer is for programming the software fo the VCU. The Ubuntu(20.04) is required and verified to work.

- This computer has the same subnet IP address with the VCU.

- The user need install the requirement software eg Filezilla(sftp client), GTKTerm(serial debug) or ssh for downloading, uploading and debugging the VCU files.

- The user run sdk.sh(from teamone) to install the sdk compiler.

- <your-sdk-dir>/environment-setup-cortexa72-cortexa53-xilinx-linux includes the enviroment setup for the compiler. The user need setup the enviroment of the compiler based on their SDK requirement.

# B3VCU system resources

## External interface

### CAM UART

This port is link to the TI camera for the NUC controlling.

Physcial link: TIA/EIA-422

Device name: /dev/ttySC0

working mode:Duplex

flow control: no

### VDU UART

This port is link to the VDU for the VDU switching command.

Format: TIA/EIA-422

Device name: /dev/ttySC1

working mode:Duplex

flow control: no

### Spare UART

Format: TIA-232

Device name: /dev/ttyPS1,  /dev/ttySC2,  /dev/ttySC3

working mode:Duplex

flow control: no

### Ethernet

10/100/1000 BASE-T

- Port 1: VCU control port. default static IP  192.168.9.115

- Port 2: VCU RTP video stream port. Default Static IP 192.168.1.111

- Port 3: VCU RTP video stream port. Default Static IP 192.168.2.112

### KeyPad

| Key No | Gpio port | Key name | Linux code |  |
| --- | --- | --- | --- | --- |
| KEY_DIN_1 | <axi_gpio_0 0> | key1 | KEY_1(2) | Trailer ? |
| KEY_DIN_2 | <axi_gpio_0 1> | key2 | KEY_2(3) | Reverse Gear |
| KEY_DIN_3 | <axi_gpio_0 2> | key3 | KEY_3(4) | Forward Gear |
| KEY_DIN_4 | <axi_gpio_0 3> | key4 | KEY_4(5) | TBD |
| KEY_DIN_5 | <axi_gpio_0 4> | key5 | KEY_5(6) | TBD |
| KEY_DIN_6 | <axi_gpio_0 5> | key6 | KEY_6(7) | TBD |
| KEY_DIN_7 | <axi_gpio_0 6> | key7 | KEY_7(8) | TBD |
| KEY_DIN_8 | <axi_gpio_0 7> | key8 | KEY_8(9) | TBD |

### Digital GPIO Output

| DO No | Linux GPIO No | Terminal | Description |
| --- | --- | --- | --- |
| DO1 | 492 | IR | On/OFF IR |
| DO2 | 493 | IR | On/OFF IR |
| DO3 | 494 | TBD |  |
| DO4 | 495 | TBD |  |
| DO5 | 496 | TBD |  |

### SDI Video Input

Device name: /dev/media6 (/dev/video0)

/dev/media7 (/dev/video1)

/dev/media8 (/dev/video2)

/dev/media9 (/dev/video3)

/dev/media0 (/dev/video4)

/dev/media1 (/dev/video5)

/dev/media2 (/dev/video6)

/dev/media3 (/dev/video7)

/dev/media4 (/dev/video8)

/dev/media5 (/dev/video9)

### SDI video Output

SDI0 output: frmbuf_rd --> mix -> uhdsdi_tx (‘mix’ can mix video and osd)

SDI1 – SDI7 output: frmbuf_rd->udhsdi_tx. (without osd funciton)

There is a device “sdi_anc_tx” in all output. It can embed timestamp into the SDI signal.

all devices refer to internal interface:

frmbuf_rd: refer to 4.2.2

mix: refer to 4.2.3

uhdsdi_tx: refer to 4.2.4

sdi_anc_tx: refer to 4.2.5

### LED

| LED | Linux GPIO No | Description |
| --- | --- | --- |
| Green | 458 | Normal Status |
| Red | 459 | BIT error |

## Internal Interface

### I2C

device name: /dev/i2c-0

slave chip: mb85rc256vfp  slave address: 0x50

description: log data storage

### frmbuf_rd

Address: sdi0(0xb02d0000) sdi1(0xb02f0000) sdi2(0xb0310000) sdi3(0xb0330000) sdi4(0xb03a0000) sdi5(0xb03c0000) sdi6(0xb03e0000) sdi7(0xb0400000)

Support Format: Y_UV10, Y_UV8

refer to document (pg278-v-frmbuf.pd) provided by Xilinx

### mix

Address: 0xb02e0000

Primary video format: YUV4:2:2 10bits

Overlay layer1 format: BGRA8

Maximum resolution: 1920x1080

refer to doucument (pg243-v-mix.pdf) provided by Xilinx

### uhdsdi_tx

- Address: sdi0(0xa0040000) sdi1(0xa0060000) sdi2(0xa0080000) sdi3(0xa00a0000) sdi4(0xa0100000) sdi5(0xa0120000) sdi6(0xa0140000) sdi7(0xa0160000)

- Mode: 3G SDI 10bits

- format: NV16_10LE32

- refer to document (pg289-v-smpte-uhdsdi-tx-ss.pdf) provided by Xilinx

### sdi_anc_tx

- Address: sdi0(0xa01c0000) sdi1(0xa01d0000) sdi2(0xa01e0000) sdi3(0xa01f0000) sdi4(0xa0200000) sdi5(0xa0210000) sdi6(0xa0220000) sdi7(0xa0230000)

- data format: 64bits unsigned interger. Microseconds of the epoch time.

### IIo sensor

CPU:/sys/bus/iio/devices/iio\:device0

Power Current Sensor:/sys/bus/iio/devices/iio\:device1

### pdma

It is a user interface to transfer a DAM address of the user space to the physical address.

Device: /dev/pdma0

ioctl cmd: 0

ioctl parameters: struct{guint64 param_in; guint64 param_out}

param_in: the dma user-space address.

param_out: the dma physical address

### Reserve memory

reserve memory physcial address:0x70000000

reserve memory size: 0xff00000

### Watchdog

Device: /dev/watchdog0, /dev/watchdog1

watchdog0 is controlled by OS.

Watchdog1 can be controoled by the user.

# Programming

## A simple firmware

### Edit

Install Atom as the SDK editor.  Install Atom requirement plugin.

Create the Project Folder and Add this project folder into the Atom.

create a new file ‘Makefile’ and a folder ‘src’ in this project folder.

Create a source file ‘main.c’ in the folder ‘src’.

edit Makefile:

TARGET := $(notdir $(CURDIR))

CSOURCES := $(wildcard src/*.c)

COBJECTS := $(CSOURCES:%.c=%.o)

CPPSOURCES := $(wildcard src/*.cpp)

CPPOBJECTS := $(CPPSOURCES:%.cpp=%.o)

CFLAGS := -O

LDFLAGS :=

LIBS :=

.PHONY: $(TARGET)

$(TARGET): $(COBJECTS) $(CPPOBJECTS)

$(CXX) -o $@ $^ $(LDFLAGS) $(LIBS)

$(COBJECTS): %.o: %.c

$(CC) $(CFLAGS) -o $@ -c $<

$(CPPOBJECTS): %.o: %.cpp

$(CXX) $(CFLAGS) -o $@ -c $<

clean:

rm -rf *~ .dep $(COBJECTS) $(CPPOBJECTS) $(TARGET)

edit the main.c:

#include <stdio.h>

int main()

{

printf(“hello world!\n”);

return 0;

}

### Compile

Open Terminal and Go to your project folder. Run “make”.

Then you get a executable file ‘<your-project-folder>’ in <your-project-folder>.

### Run firmware

- Download the executable file to /usr/local/bin in the VCU with the sftp client software

- Open GTKTerm and connect to the VCU with the USB debug port.(e.g /dev/ttyUSB0)

- Disable the original VCU firmware in /etc/init.d/loaduserapp.sh and reboot the VCU

- go to the folder /usr/local/bin and run your application